IEEE-Paper (Dominik Knauer)

Calculation of the object list:

From the detected coordinates in form of x- and y-coordinates and a detected distance, the distance from the ego-vehicle, velocity, acceleration, angle of the object and angular velocity, can be calculated. For the calculation previous values are stored and assigned to an ID. With the proportionality of the object distance from the vertical centerline and the camera angle, the script scales in metric distance.

dPixel=yPixel-wPixel/2

dPixel: distance from centerline to object

yPixel: y-coordinates of the object

wPixel: pixel format of the video horizontal

This formula divides the image into two segments (left and right segment) and calculates the distance dPixel of the object to this center axis.

kangle=dPixel/wSegment,Pixel

kangle: proportionality factor for distance and angle

wSegment,Pixel: pixel width of segment

Here is the factor kangle calculated, which corresponds to a value of -1...0 or 0...1. This indicates, which factor of the total segment width, the object is located.

ϕ=kangle* αSegment

ϕ: angle from centerline to object

αSegment: angle of view of a Segment = 45°

The angle of view from the camera, which corresponds to 90°, is used. In relation to a segment, it would be 45°. This is calculated with the factor, to get an angle ϕ related to the vertical axis.

1.Figure (CARLA-Simulator, Screenshot)

By using the angle and the direct distance, the trigonometric theorems can be used to calculate the x- and y-distance in meters (shown in 1.Figure), which are the basis for the velocity- , acceleration- , angle- , and angular velocity-calculation. The object attributes are stored over several frames to calculate time-dependent values. Velocity is calculated by the change of the x- and y-distances, the acceleration, by changing the x- and y-velocities. The angle with the Velocity in x- and y-direction and the angular velocity is calculated by time changing angles. The calculated values are related to the ego-vehicle.

The dimensioning is based on the same formulas. The formulas above are used to calculate an angle. This angle can be used, to calculate a distance in meters from the center vertical axis, for the position y of the vehicle on the y-axis.

y=hdirect*sin⁡(ϕ*π180)

y: distance from centerline to object in meters

hdirect: direct distance to object in meters

After calculating the distance of the object to the center axis of the image, it is divided by the pixel distance of the object according to the center axis. This gives a meter per pixel value kMpP, to convert the detected object dimensions in pixels, to meter.

kMpP=ydPixel

kMpP: factor meter per pixel for scaling

Via the ROS Framework, the required data such as geometry, probability, ID of the object etc. are transferred via a node in form of a message. A topic receives this message (shown in 2. Figure) for further processing of the data in RVIZ(ROS).

2. Figure (Screenshot of a ROS message)